8×8 Synchronous FIFO

Design Overview

The design implements an 8-entry × 8-bit synchronous FIFO.

- Depth: 8 entries

- Data width: 8 bits

- Storage: 8 × 8-bit memory array

- Read/write operations: Synchronous to clk

- Reset: Asynchronous, active-HIGH

- Status flags: empty and full

- Read pointer: rd_ptr[2:0]

- Write pointer: wr_ptr[2:0]

- Wrap indicators: wrap_rd, wrap_wr

The FIFO uses separate read and write pointers along with an additional wrap bit for distinguishing between the empty and full conditions when both pointers have the same value.

FIFO Interface

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| clk | Input | 1 | Clock of the system |
| rst | Input | 1 | Asynchronous active-HIGH reset |
| in | Input | 8 | Data written into FIFO |
| out | Output | 8 | Data read from FIFO |
| wren | Input | 1 | Write enable |
| rden | Input | 1 | Read enable |
| empty | Output | 1 | Indicates FIFO has no valid data. |
| full | Output | 1 | Indicates FIFO is completely full. |

FIFO Architecture

The FIFO contains a bundle of register which provides  8 locations × 8 bits

It can perform synchronous read and write operation both simultaneously as well provided the conditions are met